From: datarec@inforamp.net (Nicholas Majors)

Subject: Chapter 2 - Technicians' Guide to Hard Disks

Date: 25 Mar 1995 18:40:54 GMT

========================================================================

Chapter 2 - TECHNICIANS' GUIDE TO PC HARD DISK SUBSYSTEMS

========================================================================

copyright (c) 1992, 1995

Nicholas Majors,

DATA RECOVERY LABS

(division of Data Recovery Services Inc)    Voice : 1-416-510-6990

1315 Lawrence Avenue East - Unit 502        FAX   : 1-416-510-6992

Don Mills, Ontario, Canada M3A 3R3          Email : datarec@the-wire.com

========================================================================

Before we consider how to install, configure and maintain hard drives,

we need a basic understanding of drive construction and design concepts.

This chapter examines in some detail the parts and functional components

of hard drive subsystems.

(Note : A number of acronyms are used throughout this chapter and the

glossary for this booklet is not yet available.  Therefore, I have

attached a brief set of definitions for some of the terminology.)

HARD DRIVES AND CONTROLLERS:

A hard drive subsystem is comprised of the following components:

1. The Hard Disk, with one or more boards (PCB) attached.

2. A Controller Mechanism, either on the hard disk PCB or on the bus

adapter within the PC.

3. Bus Adapter for interfacing the controller to the host PC.

4. Cables and Connectors to link it all together.

========================================================================

THE HARD DISK:

Within a sealed enclosure (Head Disk Assembly or HDA) are one or more

rigid  platters that are "fixed" or non-removable.  These are coated

with magnetically sensitized material and data can be written to and

read from the surface by means of electromagnetic read/write heads.

When powered up, the platters are constantly rotating (except for

certain pre-programmed sleep modes) and the heads are moved back and

forth across the surface to access different locations.  This is a

sealed unit which should not be opened, except by qualified personnel in

a controlled, dust free environment.

The circuit board(s) attached to the outside of the HDA provide the

electronics needed for physical control of the motors within the sealed

unit.  They interface the source of electrical power and control signals

to the disk assembly through various connectors and cables.  Most boards

have some jumpers, dip switches and/or resistors that are used for

configuration purposes.

Functionally, these PCB's are separate from the Hard Disk Controller,

but many of the newer drives (IDE and SCSI) embed the controller chip

directly onto this board (as opposed to having it on the Bus adapter).

INSIDE THE HDA - PARTS OF A HARD DISK:

1. Disk Platter(s), separated by spacers and held together by a clamp.

2. Spindle shaft onto which platters are mounted.

3. Spindle motor for rotating the platters.

4. Electromagnetic read/write heads (usually, one per surface).

5. Access arms or armatures from which the heads are suspended.

6. Actuator for moving the arms (with heads attached).

7. Preamplifier circuitry to maximize read/write signals.

8. Air filter and pressure vent.

The Platters:

Most platters or disks are made of an aluminum alloy, though ceramic or

glass platters can also be found.  The diameter is normally 2 1/2", 3

1/2" or 5 1/4" with a hole in the center for mounting onto the spindle

shaft.  Thickness of the media can vary from less than 1/32 of an inch

to about 1/8 of an inch.

During manufacture the platters are coated with a magnetizable material.

Older drives used a ferrite compound applied by squirting a solution

onto the surface and rotating at high speeds to distribute the material

by centrifugal force.  This process left a rust colored ferrite layer

which was then hardened, polished and coated with a lubricant.

Newer drives apply the magnetic layer by plating a thin metal film onto

the surface through galvanization or sputtering.  These surfaces have a

shiny chrome-like appearance.

Spindle and Spindle Motors:

Most drives have several platters that are separated by disk spacers and

clamped to a rotating spindle that turns the platters in unison.  A

direct drive, brushless spindle motor is built into the spindle or

mounted directly below it. (Sometimes this motor is visible from outside

of the sealed enclosure.) The spindle, and consequently the platters,

are rotated at a constant speed, usually 3,600 RPM, though newer models

have increased that to 4800, 5400, or 7,200.

The spindle motor receives control signals through a closed loop

feedback system that stabilizes to a constant rotation speed.  Control

signals come from information written onto the surface(s) during

manufacture or with older drives, from physical sensors.

Read/Write Heads:

Since both sides of each platter are coated to provide separate

surfaces, there is normally one electromagnetic read/write head for each

side of each platter.  Therefore, a drive with 4 platters would have 8

sides and 8 heads.  Some drives use one side as a dedicated surface for

control signals leaving an odd number (5,7,etc.) of heads for actual

use.

Each head is mounted onto the end of an access arm and these arms (one

per surface) are moved in unison under the control of a single actuator

mechanism.  When one head is over track 143, all the heads on all other

sides should be at the same location over their respective surfaces.

Generally speaking, only one of the heads is active at any given time.

There are some drives that can read or write from two or more heads at a

time, but while this has been common with main frame drives, it

represents a major design change for personal computers and the

technology is not yet widely used.

The spinning disk(s) create an air cushion over which the heads float.

Depending on design, this air buffer ranges from 2 to 15 microns.  By

contrast, a smoke particle or finger print is about 30 microns in size!

The heads are not supposed to come into contact with the surface during

rotation.  Only when powered off should the heads come to rest on the

surface, but this should be over a specific area of the surface,

reserved for that purpose.  Most drives built since the late 1980's

employ an automatic parking feature which moves the heads to this

designated region and may even lock the heads there until powered up.

Head Actuators:

The head actuator is the positioning mechanism used to move the arms and

consequently the heads, back and forth over the surface.  Once again,

earlier drives used a different method than is now common.

Originally, head positioning was controlled by a stepper motor that

rotated in either direction by reacting to stepper pulses and moving the

head assembly back and forth by means of a "rack and pinion" or by

spooling and unspooling a band attached to the actuator arms.  Each

pulse moved the assembly over the surface in predefined steps or

detents.  Each step represented a track location and data was expected

to be under the head.  This design, still used for floppy drives, is not

suitable for current drive densities and is prone to alignment problems

caused by friction, wear and tear, heat deformation, and lack of

feedback information needed for correcting positioning error.

The more common voice coil actuator controls the movement of a coil

toward or away from a permanent magnet based upon the amount of current

flowing through it.  The armatures are attached to this coil and move in

and out over the surface with it.  This is a very precise method, but

also very sensitive.  Any variation in the current can cause the head

assembly to change position and there are no pre-defined positions.

Inherently this is an analog system, with the exact amount of movement

controlled by the exact amount of current applied.

The actual position of the coil is determined by servo (or indexing)

information, which is written to the drive by the manufacturer.

Location is adjusted to different tracks by reading and reacting to

these control signals.

Internal Electronics:

There is surprisingly little circuitry found within the sealed HDA.

There are electrical and control wires for the spindle and head actuator

motors and the head assembly has flex cables with a preamplifier chip

often built onto it.  This chip takes pulses from the heads (as close to

the source as possible) and cleans up and amplifies these signals before

transmission to components outside of the housing.

Air Filtering and Ventilation:

Minor wear of internal components and occasional contact of the heads

with the surface can cause microscopic particles to be loosened within

the HDA.  A permanent air filter is mounted within the air stream to

remove these particles before they can cause damage to delicate

mechanisms.

Most drives also have a small vent to allow for minor air exchange from

outside of the housing.  This allows for equalization of air pressure so

drives can be used in different environments without risk of imploding

or exploding.

========================================================================

CONTROLLERS AND BUS ADAPTERS:

The hard disk controller provides the logical link between a hard disk

unit and the program code within the host computer.  It reacts to

requests from the computer by sending seek, read, write, and control

signals to the drive and must interpret and control the flow of data.

Data moving to and from the drive includes sector ID's, positioning

information and timing or clock signals.  The controller must encode,

decode and separate this control information from actual data written to

or read from the drive.

Also, data is sent to and from the drive serially, in bit format, but

the CPU wants to recieve at least a byte (8 bits) at a time.  The

controller must take bits (8 - 16 - or 32 at a time) and assemble them

into bytes, words, and doublewords that can be transferred to/from the

computer.

"OUR INDUSTRY MUST LOVE STANDARDS - WE HAVE THOUSANDS OF THEM!"

And so it is with hard disk controllers.

Controllers can be categorized in several different ways, by :

Basic computer design (PC/XT vs AT-286-386-486,etc)

- as mentioned in the first chapter, standard AT controllers use

different I/O addresses, IRQ and employ PIO as opposed to DMA.

Bus Architecture (8-16 bit ISA, 32 bit MCA/EISA/VLB/PCI, etc.)

- The adapter must be designed to interface with and use features of

available expansion spots in the host computer.

Controller Card vs Adapter

- The expansion board that plugs into the PC is commonly referred to

as a controller card, but for many drives (primarily IDE and SCSI)

the controller mechanism is built directly onto the drive PCB and

the expansion board in the PC (or built into motherboard) is

actually a Host/Bus adapter.

TROUBLESHOOTING TIP - If the BIOS reports "HDD CONTROLLER FAILURE" don't

assume the problem is with your AT/IO board.  It might well be the drive

PCB that has failed.

Controller/Drive Interface

- Both drive and controller must communicate in the same 'language'

and several different standards for electrical properties and

logical meaning of signals have been established.  These

include ST506/412, ESDI, SCSI, IDE(ATA/XTA) and EIDE(ATA2).

Data Encoding Method

- Determines how densely data can be packed onto a track. MFM

encoding is sufficient for only 17 x 512 byte sectors per track.

RLL permits up to 27 and variations of ARLL allow 34 or more

sectors per track.  This recording density is a major determinant

of storage capacity, and with rotation speed and interleave are

critical factors for true data transfer capability.

Support for Translation

- Some controllers present different logical parameters to the PC

than the actual physical geometry of the drive.

Need for ROM Extension or Software Device Driver

- Additional program code is used to provide support for hard drives

when none exists (as in PC/XTs), to implement translation schemes

(as in ST506/RLL and ESDI designs), allow for non-standard devices

or features (SCSI), or for a combination of these (EIDE).

Below is a quick list of the major combinations that have been used in

PCs past and present.  While I am sure many others could be added, these

are the ones I have come across over the years.

------------------------------------------------------------------------

Computer   Bus Connection        Interface    Encoding   Translate  ROM

------------------------------------------------------------------------

PC/XT   8 bit ISA Controller    ST506/412      MFM       NO       YES

PC/XT   8 bit ISA Controller    ST506/412      RLL       OPTION   YES

AT     16 bit ISA Controller    ST506/412      MFM       NO       NO

AT     16 bit MCA Controller    ST506/412      MFM       NO       NO

AT     16 bit ISA Controller    ST506/412      RLL       OPTION   YES

AT     16 bit MCA Controller    ST506/412      RLL       YES      YES

AT     16 bit ISA Controller *  ESDI (10 Mbps) RLL       OPTION   YES

AT     16 bit ISA Controller *  ESDI (24 Mbps) ARLL      OPTION   YES

AT     16 bit MCA Controller ** ESDI (PS/2)    RLL,ARLL  YES      YES

PC/XT   8 bit ISA Adapter       SCSI           RLL       YES      YES

AT     16 bit ISA Adapter       SCSI           RLL,ARLL  YES      YES

AT     ?? bit MCA Adapter ***   SCSI           RLL,ARLL  YES      YES

AT     32 bit EISA Adapter      SCSI           RLL,ARLL  YES      YES

AT     32 bit VLB Adapter       SCSI           RLL,ARLL  YES      YES

AT     32 bit PCI Adapter       SCSI           RLL,ARLL  YES      YES

PC/XT   8 bit ISA Adapter       IDE  / XTA     RLL       OPTION   YES

AT     16 Bit ISA Adapter       IDE  / ATA     RLL,ARLL  OPTION   NO

AT     32 Bit VLB Adapter       EIDE / ATA2    ARLL      OPTION   YES

AT     32 Bit PCI Adapter       EIDE / ATA2    ARLL      OPTION   YES

*   ESDI drives have some of the controller logic built onto

the hard drive PCB and some on the controller card.

**  PS/2 ESDI uses the same physical interface as other ESDI

devices, but supports additional features specific to their

implementation.

*** 16 bit? 32 bit? Who knows?  I have never been sure.

------------------------------------------------------------------------

Not to mention hundreds of other combinations to support different

interleaves, track buffers, hardware caching, bus mastering, error

correction schemes, SCSI I-II-III, optional floppy control, ESDI to SCSI

converters,  ST506 to SCSI converters (etc., etc., etc.).

So, what does all this mean to you?

Specifically, don't be surprised if the drive you have in your left

hand, does not work correctly with the controller / adapter you have in

your right hand.   Also, if controllers are changed it may affect

performance as well as the ability to access previously recorded data.

========================================================================

END OF CHAPTER 2

========================================================================

How to deal with some of this confusion will be addressed in

CHAPTER 3 - Installing and Configuring Hard Drives.

========================================================================

ADDITIONAL READING MATERIAL:

========================================================================

I have always had difficulty finding appropriate reading material to

recommend, but there are a few sources that I consider a must for

technicians and support personnel.

First, a number of Internet Newsgroups have exceptional FAQ's

(Frequently Asked Questions) which are updated and posted on a regular

basis.  These include:

"YET ANOTHER ATA-2/FAST-ATA/EIDE FAQ"

by John Wehman and Peter Herweijer

Newsgroup : comp.sys.ibm.hardware.storage

"BIOS TYPES"

by Hale Landis

Newsgroup : comp.sys.ibm.hardware.storage

"SCSI FAQ - 2 Parts"

by Gary A. Field

Newsgroup : comp.periphs.scsi

While bookstores are full of titles, most of them simply provide a

rehash of basics.  The following two books are an important part of my

library because they cover much more than the usual:

"THE INDISPENSABLE PC HARDWARE BOOK" by Hans-Peter Messmer (1994)

Addison-Wesley Publishing Company,

ISBN - 0-201-62424-9

"THE UNDOCUMENTED PC" by Frank Van Gilluwe (1994)

Addison-Wesley Publishing Company,

ISBN - 0-201-62277-7

Other books well worth the read include:

"THE HARD DISK SURVIVAL GUIDE" by Mark Minasi (1991)

Sybex Inc.,

ISBN - 0-89588-799-1

A little dated, but full of useful information.  Hopefully there is a

revised and updated version.

"OFFICIAL SPINRITE II AND HARD DISK COMPANION" by J. M. Goodman, (1990)

IDG Books Worldwide, Inc.,

ISBN - 878058-08-8

Current advances are not covered, but great explanation of drive basics.

"HARD DRIVE BIBLE" by Martin Bodo (1993)

Corporate Systems Center (CSC)

My copy is the sixth edition from April, 1993.  The first 50 pages of

the book should be of interest (though not always clearly organized).

The balance of the book (150 pages) is a listing of drive types and

jumper settings.  It's quite good, but keeping something like that

updated is virtually impossible.

WARNING - DO NOT BUY - "The Data Recovery Bible" by Pamela Kane.

Poorly organized material, most of which has nothing to do with data

recovery.  Waste of a good title if you ask me!

========================================================================

ACRONYM DEFINITIONS:

========================================================================

IRQ (Interrupt Request) - Lines on the bus used to signal hardware

interrupts.

I/O (Input Output) - Peripherals accessible by the CPU through registers

at specific I/O addresses (or I/O ports).

PIO (Programmed Input Output) - Exchange of data between memory and

peripherals by means of Input Output commands.

DMA (Dynamic Memory Access) - Transferring data directly between memory

and peripherals without going through the CPU.

BUS ARCHITECTURES:

ISA (Industry Standard Architecture) - 8 bit and 16 bit expansion slots

used by PC, XT, and AT designs.  Often called IBM Standard Architecture.

EISA (Extended Industry Standard Architecture) - Developed by several

independent manufacturers (Compaq, AST, Zenith, Tandy, etc.) to

standardize 32 bit operation and combat IBM's MCA.

MCA (Micro Channel Architecture) - Expansion bus introduced by IBM in

1987, used by some (but not all) PS/2 models.

PCI (Peripheral Component Interconnect) - High speed bus developed by

Intel to support the demands of Pentium and 486 based computers.

VLB (VESA Local Bus) - High speed, 32 bit extension to the ISA bus

promoted by the VESA (Video Electronics Standards Association).

DRIVE INTERFACES:

ST506/412 - Standard interface used on XT and AT drives and controllers.

Originally developed by Seagate Technologies to support their ST506

(5 MB) and ST412 (10 MB) drives.  The entire controller mechanism is

located on a controller card and communications between the drive and

controller flow over 2 ribbon cables - one for drive control and one for

data.

ESDI (Enhanced Small Device Interface) - Developed by Maxtor in the

early 1980's as an upgrade and improvement to the ST506 design.  While

the drive does not have an embedded controller, one of the most critical

functions ,encoding-decoding, is performed on the drive.  This allows

for faster communications and higher drive capacities.  Uses the same

cabling as ST506 interface, but carries different signals on each line.

SCSI (Small Computer System Interface) - Based on an original design by

Shugart Associates, SCSI is not specifically a drive interface, but a

method of allowing different devices to communicate with a PC.  For hard

drives the entire controller is built onto the drive PCB, allowing for

very high speed transfers to and from the drive.  Fully interpreted,

parallel data is then transferred to and from the PC by way of a single

cable through a bus interface that has configured the device as a hard

drive.

IDE (Integrated Drive Electronics) - A technology pioneered by Compaq

and Conner that embedded a controller onto the hard disk PCB while

maintaining compatibility with the register level commands sent by the

computer's INT 13 routines.  IDE drives are configured and appear to the

computer like standard ST506 drives.

ATA (AT Attachment) - Implementation of the IDE design with a 16 bit AT

style controller on board the drive.

XTA (XT Attachment) - Rarely used implementation of IDE with an

integrated 8 bit XT controller.

ATA-2 - Enhancement to the AT Attachment standard to provide for

considerable performance improvement and more sophisticated drive

identification.

EIDE (Enhanced IDE) and FAST-ATA - Various implementations of the ATA-2

standard as marketed by Western Digital (EIDE) and Seagate/Quantum

(FAST-ATA).

DATA ENCODING SCHEMES

MFM (Modified Frequency Modulation) - Common technique used to encode

the magnetic fluxes recorded on a drive into data.  Still used on floppy

drives and most original XT and AT systems.  Notice that most drive

types supported by the motherboard BIOS have 17 sectors per track.  This

is the standard density for MFM encoding.

RLL (Run Length Limited) - Encoding method that allows 50% more

information to be recorded on a track than MFM.  Accomplished by

recording larger representations for every byte, but able to pack them

more tightly onto the surface, because of fewer actual flux changes.

Often called 2,7 RLL because the recording scheme involves patterns with

no more than 7 successive zeros and no less than two.

ARLL (Advanced Run Length Limited) - More complex yet powerful

derivatives of the RLL scheme.  Include 1,7 and 3,9 encoding.  Most

every new drive made today uses some form of RLL or ARLL encoding.

========================================================================